NETWORK TROUBLESHOOTING GUIDE

Professional IT Network Support Documentation

| Prepared By | Britany Walker |
| --- | --- |
| Document Type | Professional IT Network Support Documentation |
| Version | 2.1 |
| Effective Date | May 2026 |
| Classification | Original Portfolio Document - No Client or Employer Data |

Portfolio Notice: This document is an original professional sample created for portfolio use. It does not include client data, employer data, confidential information, passwords, private identifiers, or proprietary system details.

# Table of Contents

| Section | Title |
| --- | --- |
| 1 | Purpose |
| 2 | Scope |
| 3 | Initial Triage Checklist |
| 4 | Network Decision Tree |
| 5 | Diagnostic Commands |
| 6 | Wired and Wireless Troubleshooting |
| 7 | DNS Troubleshooting |
| 8 | VPN Connectivity Troubleshooting |
| 9 | Escalation Criteria |
| 10 | Ticket Documentation Standards |
| 11 | Security Considerations |
| 12 | Quality Review Checklist |
| 13 | Revision History |

# 1. Purpose

This guide gives IT support staff a clear, practical process for troubleshooting common network connection issues. It helps technicians identify the source of a problem, collect useful diagnostic information, reduce downtime, and escalate issues with the right details when advanced support is needed.

# 2. Scope

This guide covers common wired network, wireless network, DNS, TCP/IP, VPN, and general connectivity issues reported by end users, workstations, laptops, and business applications. It is designed as a general support reference and does not include confidential network architecture or client-specific information.

# 3. Initial Triage Checklist

When a network issue is reported, start by gathering the basic details. Confirm who is affected, what device they are using, and where the device is located. This helps identify whether the problem is tied to a specific user, device, area, or network segment.

Next, determine the scope of the issue. Ask whether it affects one user, multiple users, one application, one device, or an entire location. This helps determine the urgency and whether the issue should be escalated.

Also confirm the connection type. The troubleshooting path will be different depending on whether the user is connected through Ethernet, Wi-Fi, VPN, or a mobile hotspot.

Ask when the issue started and whether anything changed recently, such as a device move, update, outage, password change, or configuration change. Finally, collect any available evidence, such as error messages, screenshots, or command results. Good evidence makes the ticket easier to understand and helps the next support team respond faster.

| Triage Item | Question to Answer | Why It Matters |
| --- | --- | --- |
| User and device | Who is affected, what device is being used, and where is it located? | Confirms ownership and helps identify location-based patterns. |
| Scope | Is this one user, several users, one application, one device, or a full site issue? | Helps determine urgency and escalation needs. |
| Connection type | Is the user on Ethernet, Wi-Fi, VPN, or a mobile hotspot? | Narrows the troubleshooting path. |
| Start time | When did the issue begin, and what changed recently? | Links the issue to updates, outages, moves, or configuration changes. |
| Evidence | What error message, screenshot, or command result is available? | Supports cleaner notes and faster escalation. |

# 4. Network Decision Tree

Start by checking whether the device can connect to any network at all. If it can, continue testing DNS and application access. If it cannot, review the adapter, cable, Wi-Fi settings, or device configuration.

If multiple users are affected, check for outage notices and escalate to network support. If only one device or user is affected, continue with device-level troubleshooting.

If the device can reach an IP address but cannot reach a hostname, investigate DNS settings and name resolution. If it cannot reach either one, test the gateway, routing, and packet loss.

If the VPN connects but internal resources still do not load, check DNS suffixes, routing, permissions, and internal host access. If the VPN does not connect, verify the VPN profile, MFA, credentials, and client version.

| Step | Decision Point | If Yes | If No |
| --- | --- | --- | --- |
| 1 | Can the device connect to any network? | Test DNS and application access. | Check the adapter, cable, Wi-Fi settings, or device configuration. |
| 2 | Are multiple users affected? | Check outage notices and escalate to network support. | Continue device-level diagnostics. |
| 3 | Can the device reach an IP address but not a hostname? | Investigate DNS settings and name resolution. | Test gateway, routing, and packet loss. |
| 4 | Does VPN connect while internal resources fail? | Check DNS suffixes, routing, permissions, and internal host access. | Validate VPN profile, MFA, credentials, and client version. |

# 5. Diagnostic Commands

Use these commands to collect evidence before escalating. Record the result clearly so another technician can understand what was tested and what changed.

| Command | Purpose | Example | What to Record |
| --- | --- | --- | --- |
| ping | Tests basic connectivity to a host. | ping 8.8.8.8 | Success or failure, packet loss, and response time. |
| ipconfig /all | Reviews IP address, DNS server, gateway, and adapter details. | ipconfig /all | IP address, gateway, DNS servers, and adapter status. |
| ipconfig /flushdns | Clears the local DNS cache. | ipconfig /flushdns | Whether name resolution improves after clearing cache. |
| tracert | Shows the network path to a destination. | tracert 8.8.8.8 | Where the route fails or times out. |
| nslookup | Tests DNS name resolution. | nslookup example.com | Resolved IP address or DNS error message. |
| netsh wlan show interfaces | Shows wireless connection details. | netsh wlan show interfaces | SSID, signal strength, radio type, and connection state. |

# 6. Wired and Wireless Troubleshooting

For wired Ethernet issues, confirm that the cable is securely connected, check link lights, test with a known-good cable or port, restart the adapter, and confirm that the device has a valid IP address. Escalate if there is no link light, the device receives an invalid IP address, packet loss continues, or multiple users are affected.

For wireless issues, confirm that Wi-Fi is enabled, verify that the user is connected to the correct SSID, check signal strength, forget and reconnect to the network, restart the adapter, and test another device if possible. Escalate if authentication keeps failing, the access point appears offline, or several users in the same area are affected.

For VPN issues, confirm that the user has working internet access before troubleshooting the VPN client. Then verify the VPN profile, credentials, MFA, client version, and any error codes. Escalate if the user does not have VPN permissions, MFA repeatedly fails, or internal resources remain unavailable after a successful VPN connection.

# 7. DNS Troubleshooting

Confirm that the workstation has the expected DNS server settings. Use nslookup to check whether hostnames are resolving correctly. Clear the DNS cache with ipconfig /flushdns, then retest the affected site or application.

Test access by both hostname and IP address. This helps determine whether the problem is related to DNS or general connectivity. Escalate repeated DNS failures, incorrect DNS assignments, or organization-wide name resolution issues.

# 8. VPN Connectivity Troubleshooting

Before troubleshooting the VPN client, confirm that the device has working internet access. Then verify that the VPN client is installed, updated, and configured with the correct profile or server address.

Validate the username, password, MFA approval, and VPN permissions. If the client is frozen, stuck connecting, or repeatedly timing out, restart the VPN application and the workstation.

Document VPN error codes, timestamps, failed authentication attempts, and any internal resources tested. This information is especially important if the issue needs to be escalated.

# 9. Escalation Criteria

| Escalate When | Escalate To | Required Notes |
| --- | --- | --- |
| Multiple users or locations are affected. | Network Administrator / Infrastructure Team | Affected users, location, start time, outage symptoms, and current workaround. |
| DNS resolution fails across multiple devices. | Network or Systems Team | nslookup results, DNS settings, tested hostnames, and impacted applications. |
| VPN authentication fails after credential and MFA checks. | Identity / Access Management Team | Username, error message, MFA status, timestamps, and account status if known. |
| Packet loss, routing failure, or gateway issues continue. | Network Administrator | ping/tracert results, gateway IP, device name, and connection type. |

# 10. Ticket Documentation Standards

Document the user impact, affected device, network type, location, and business urgency.

Include command outputs, screenshots, error messages, and test results when available.

List troubleshooting steps in the order completed so another technician can follow the work already done.

Keep ticket status, priority, escalation path, owner, and final resolution clear.

Do not record passwords, MFA codes, recovery codes, or unnecessary personal information in the ticket.

# 11. Security Considerations

Use approved remote support tools and follow identity verification procedures before making account or device changes. Do not connect unauthorized personal devices to a business network during troubleshooting.

Report suspicious traffic, repeated lockouts, impossible travel alerts, or unauthorized access attempts right away. If a security issue is suspected, preserve evidence and avoid making unnecessary changes before escalation.

# 12. Quality Review Checklist

| Review Item | Standard | Status |
| --- | --- | --- |
| Triage complete | User, device, scope, location, and start time are documented. | ☐ Pass ☐ Needs Review |
| Diagnostics recorded | Relevant ping, ipconfig, tracert, nslookup, or VPN error details are included. | ☐ Pass ☐ Needs Review |
| Escalation quality | Ticket includes clear handoff notes and business impact. | ☐ Pass ☐ Needs Review |
| Security safe | No passwords, MFA codes, confidential network diagrams, or private identifiers are included. | ☐ Pass ☐ Needs Review |

# 13. Revision History

| Version | Date | Description of Change | Author |
| --- | --- | --- | --- |
| 2.1 | May 2026 | Merged humanized content, cleaned wording, preserved professional support structure, and improved readability. | Britany Walker |